ENEE452 Midterm Plan of Action - Morse Code Trainer

Tyler Wong

March 12, 2026

# 1) Serial I/O

Configure USART with RX interrupt enabled. Write an RX ISR that appends each received byte to a circular buffer.

In main, write a readline() function that pulls from the circular buffer into a char array until it sees \r or \n. Also write a parse_command() function that checks whether the line matches D<nnn> structure, and if so extract the integer to validate the 20-300 range, then return speed t. Also write uart_print() to send strings back to the terminal.

Interim tests:

- Flash with program that echoes every received character back

- Send D60 and verify that parse_command() returns 60

Subgoal: MCU received a valid command, parses, and returns output marking the start of the decoding session and speed t.

# 2) Waveform Generation

Configure PA5 as TIM2 function output. Configure TIM2 so that its frequency is 800Hz. Write tone_on() and tone_off() functions to enable and disable this output.

Interim tests:

- Call tone_on() and verify 800Hz

- Call tone_off() and verify PA5 stays low

Subgoal: 800Hz square wave that can be turned on and off.

# 3) Timing Control

Write a scaling function that takes the _ABCsp[] array and t and outputs a scaled copy ditdah_t[] with values in milliseconds.

Dits/dahs duration controlled by TIM3, write TIM3 ISR as a step function – read the next element from the array, check its sign to determine whether to call tone_on() or tone_off(), load |value| * (timer_clock / 1000) into register, then restart the timer. When the index exceeds the last element, ISR calls tone_off(), stops TIM3, and sets a finished flag.

Write a start_sequence() function that resets index to zero, loads the first element, and starts the timer. This is called after command is parsed.

Interim Tests:

- Run _ABCsp[] sequence with t = 60 and verify timing against the expected pattern.

- Verify that the finished flag is set only once and that main detects it without polling in an infinite loop.

Subgoal: Run “ABC “ sequence back to back by calling start_sequence() each time finished flag is detected.

# 4) Check Answer

In main, after detecting the finished flag, output a prompt to terminal for the user to enter their translation. Use readline() to capture the user input.

Write check_answer() function that takes the user input string and the reference string (“ABC “). It should iterate through each character, printing each position with a pass/fail. When it is done, it should print a summary line showing how many characters were correct.

Interim Tests:

- Test check_answer() by itself before adding it to the whole program. Call it with both incorrect and correct user input strings and verify correct output.

- Add it to the whole program and verify it works with D60 command and “ABC “ with correct output.

- Verify that a new session starts after the results are printed.

Subgoal: Complete operation of the gizmo.