ENEE452 Midterm Testing Instructions - Morse Code Trainer

Tyler Wong

March 12, 2026

# Setup

Connect flashed board via USB and open a serial terminal (Realterm or PuTTY). Set port to 115200 baud. Select the correct COM port.

# Instructions

Type a valid command (ex. “D60”). Hit ‘Enter’. MCU should echo a confirmation message and start sending the morse code sequence.

Invalid commands like “D1” and “D999” and “ “ can be used for error handler testing.

With command “D60”, observe the tone and note the 800Hz wave:

‘A’ : dit(60ms), gap(60ms), dah(180ms)

Silence(180ms)

‘B’ : dah, gap, dit, gap, dit, gap, dit

Silence

‘C’ : dah, gap, dit, gah, dah, gap, dit

Trailing ‘space’ silence (420ms)

Total sequence time should be ~3.5 seconds.

After the sequence is played, type the decoded answer and press ‘Enter’. Type “ABC “ to make sure answer checking confirms that the input is totally correct.

For mistake detection, mistype a character(s), try answering without the trailing space, or typing nothing.